# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

Contract Title: “Supply of IT Equipment and Software for EUAM Ukraine”

Publication Reference: EUAM-26-07, Lot 1 “Supply of IT Equipment and Standard Software”

Column 1-2 should be completed by the Contracting Authority

Column 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority, which shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient),

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation.

Voltage and plug requirements: All Grid powered devices must be compatible with standard Ukrainian voltage (220-230 volts), support 50 Hz current frequency and have C (European) or F (Schuko) plug type.

Official Warranty: All provided IT equipment must be covered by the official manufacturer’s warranty, fully valid and enforceable within the territory of Ukraine. The warranty period is minimum 12 months after delivery and acceptance of the equipment or manufacturer’s standard warranty period, if longer than 12 months.

After sale support: The Tenderer shall demonstrate the capacity to coordinate after-sales support through a dedicated technical department or an authorized service partner located in Ukraine.

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Technical offers that do not permit to identify precisely the models, their type, brand, article numbers and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications. 
Link to the website where the description of the proposed model exists is not acceptable. It is obligatory to add the detailed description of specification offered.

Where the tenderer has offered an “equivalent” item to a brand and model indicated in these technical specifications, it will be the tenderer’s responsibility to prove the claimed equivalence, for example by appending to his offer for the respective item a comparison table with the features and functionalities of the item mentioned by its brand name and model in these specs and the corresponding features and functionalities of the product offered by the tenderer.

The Contractor shall create a web-based online catalogue. The created online catalogue shall be accessible only to the Contracting Authority and the Contractor. Only the items included in the Contractor’s offer (Technical Offer (Annex III) and Budget Breakdown (Annex IV)), shall be available in the aforementioned individualised online catalogue, and shall be specified there by their brands, models, and respective unit prices, mirroring the brands, models and unit prices in the framework contract. Any new items added into the framework contract as per Article 22 of the general conditions or further to the provisions of article 1.1.5 of the framework contract, shall simultaneously be added to the existing online catalogue. The items included into the online catalogue can be modified, replaced or removed only by an explicit written agreement of the parties to the framework contract (by a formal administrative order issued by the Project Manager following the applicable contractual clauses).

| 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- |
| Item number | Specifications Required | Specifications Offered (Brand, model, technical characteristics of the offered item) | Notes, remarks, ref to documentation | Evaluation Committee’s Notes |
| 1 | COMPUTER SYSTEMS |  |  |  |
| 1.1 | DESKTOP COMPUTERS (MINIMUM REQUIREMENTS) |  |  |  |
| 1.1.1 | Mini Workstation Desktop: Small Form Factor or Mini Tower form factor; processor not less than Intel Core i3 or AMD Ryzen 3 class; memory not less than 32 GB DDR5; graphics adapter with not less than 8 GB dedicated VRAM; storage not less than 1 TB NVMe SSD, with availability for not less than one additional internal storage slot; support for not less than two simultaneous digital displays; video outputs not less than 2 × digital outputs (HDMI and/or DisplayPort); integrated Gigabit Ethernet (RJ-45); wireless connectivity Wi-Fi 6 or Wi-Fi 6E and Bluetooth 5.2 or higher; ports not less than 4 × USB Type-A, not less than 1 × USB Type-C, and not less than 1 × audio combo jack; operating system Windows 11 Pro OEM pre-installed. |  |  |  |
| 1.1.2 | PC Desktop (Standard): Tower or Small Form Factor; processor not less than Intel Core i5 or AMD Ryzen 5 class, from the latest two generations; memory not less than 32 GB RAM; integrated graphics adapter; storage not less than 1 TB SSD; support for not less than two simultaneous digital displays; video outputs not less than 2 × digital outputs (HDMI and/or DisplayPort); integrated Gigabit Ethernet (RJ-45); wireless connectivity Wi-Fi 6 or Wi-Fi 6E and Bluetooth 5.2 or higher; ports not less than 4 × USB Type-A, not less than 1 × USB Type-C, and not less than 1 × audio combo jack; operating system Windows 11 Pro OEM pre-installed. |  |  |  |
| 1.1.3 | PC Desktop (Advanced): Tower form factor; processor not less than Intel Core i7 or AMD Ryzen 7 class, from the latest two generations; memory not less than 64 GB RAM; graphics adapter with not less than 12 GB dedicated VRAM; storage not less than 1 TB NVMe SSD, with availability for not less than two internal storage slots; support for not less than three simultaneous digital displays; video outputs not less than 3 × digital outputs (HDMI and/or DisplayPort); integrated Gigabit Ethernet (RJ-45); wireless connectivity Wi-Fi 6 or Wi-Fi 6E and Bluetooth 5.2 or higher; power supply not less than 750 W; ports not less than 4 × USB Type-A, not less than 1 × USB Type-C, and not less than 1 × audio combo jack; operating system Windows 11 Pro OEM pre-installed. |  |  |  |
| 1.1.4 | PC Desktop (NextGen): Tower form factor; processor not less than Intel Core i9 or AMD Ryzen 9 class, from the latest two generations; memory not less than 64 GB DDR5; graphics adapter with not less than 16 GB dedicated VRAM; storage not less than 2 TB NVMe SSD; support for not less than three simultaneous digital displays; video outputs not less than 3 × digital outputs (HDMI and/or DisplayPort); integrated 2.5 Gb Ethernet; wireless connectivity Wi-Fi 6 or Wi-Fi 6E and Bluetooth 5.2 or higher; power supply not less than 1000 W; ports not less than 4 × USB Type-A, not less than 2 × USB Type-C, and not less than 1 × audio combo jack; operating system Windows 11 Pro OEM pre-installed. |  |  |  |
| 1.1.5 | All-in-one PC Desktop (Monoblock): Display not less than 27-inch with resolution not less than QHD (2560×1440); processor not less than Intel Core i7 or AMD Ryzen 7 class, from the latest two generations; memory not less than 32 GB RAM; storage not less than 1 TB SSD; integrated graphics adapter; integrated camera not less than Full HD (1080p); integrated microphone and speakers; keyboard and mouse included; support for not less than two simultaneous digital displays where supported by the offered configuration; integrated Gigabit Ethernet (RJ-45); wireless connectivity Wi-Fi 6 or Wi-Fi 6E and Bluetooth 5.2 or higher; ports not less than 2 × USB Type-A, not less than 1 × USB Type-C, and not less than 1 × video output where supported by the offered configuration; operating system Windows 11 Pro OEM pre-installed. |  |  |  |
| 1.2 | DESKTOP ACCESSORIES & SPARES (MINIMUM REQUIREMENTS) |  |  |  |
| 1.2.1 | Web camera (Standard): Resolution not less than 4K at 30 fps; HDR; autofocus; integrated stereo microphones; privacy shutter; universal monitor mounting clip or stand attachment; USB connectivity. |  |  |  |
| 1.2.2 | Web Camera (Full HD): Resolution not less than 1920 × 1080; autofocus; USB-C connectivity; universal monitor mounting clip or stand attachment. |  |  |  |
| 1.2.3 | Compact Wired Speaker System: Stereo wired speaker system; output power not less than 10 W RMS. |  |  |  |
| 1.2.4 | External 3.5-inch Desktop Hard Drive: Storage capacity not less than 14 TB HDD; USB-C interface; external power supply. |  |  |  |
| 1.2.5 | Portable HDD: Storage capacity not less than 2 TB; USB-C interface; USB-powered. |  |  |  |
| 1.2.6 | Portable HDD High Capacity: Storage capacity not less than 5 TB; USB-C interface; USB-powered. |  |  |  |
| 1.2.7 | External Secure SSD (USB-A): Storage capacity not less than 2 TB; data transfer speed not less than 450 MB/s; AES encryption; USB-A interface. |  |  |  |
| 1.2.8 | External Secure SSD (USB-C): Storage capacity not less than 2 TB; USB-C Gen2 interface; AES encryption. |  |  |  |
| 1.2.9 | Standalone Virtual Reality (VR) headset: Standalone VR headset; refresh rate not less than 120 Hz; inside-out tracking; wireless connectivity Wi-Fi 6E or higher; internal storage not less than 128 GB. |  |  |  |
| 1.3 | LAPTOP COMPUTERS (MINIMUM REQUIREMENTS) |  |  |  |
| 1.3.1 | Basic Laptop (Qwerty EN) Display not less than 17-inch with resolution not less than Full HD; processor not less than Intel Core i5 or AMD Ryzen 5 class, from the latest two generations; memory not less than 32 GB RAM; storage not less than 1 TB SSD; integrated Gigabit Ethernet or manufacturer-supported adapter; wireless connectivity Wi-Fi 6 or higher; TPM 2.0; integrated camera not less than 720p; integrated microphone and speakers; weight not more than 3.0 kg; operating system Windows 11 Pro OEM pre-installed. |  |  |  |
| 1.3.2 | Professional 16-inch Business Laptop (Qwerty EN) Processor not less than Intel Core i5 or AMD Ryzen 5 class, from the latest two generations, with not less than 6 cores and 12 threads or equivalent; operating system Windows 11 Professional 64-bit OEM pre-installed; not less than 16 GB DDR4 or DDR5, installed in SO-DIMM modules, with at least two physical SO-DIMM slots, of which at least one slot shall be free (unoccupied) at delivery; storage not less than 1 TB M.2 NVMe PCIe SSD; display not less than 16.0-inch diagonal, WUXGA (1920 × 1200) or higher, 16:10 aspect ratio, anti-glare; brightness not less than 300 nits; integrated Gigabit Ethernet (RJ-45); wireless connectivity Wi-Fi 6 or Wi-Fi 6E and Bluetooth 5.2 or higher; ports not less than 1 × USB Type-C, not less than 2 × USB Type-A (USB 3.2 Gen 1 or higher), not less than 1 × HDMI 2.1 or higher, and not less than 1 × 3.5 mm combo audio jack; webcam not less than 720p with privacy shutter; integrated hardware TPM 2.0; Kensington Nano or equivalent security lock slot; full-size backlit English International keyboard (QWERTY) with dedicated numeric keypad; battery capacity not less than 60 Wh with fast-charge support; power adapter not less than 65 W; weight not more than 2.0 kg. |  |  |  |
| 1.3.3 | Professional 16-inch High-Performance Mobile Workstation (Qwerty EN) Processor not less than Intel Core Ultra 7 or 9, Intel Core i7 or i9 HX-series, or AMD Ryzen 7 or 9 class; minimum 16 threads or equivalent; graphics adapter with not less than 8 GB GDDR6 VRAM and support for ISV-certified applications; memory not less than 32 GB DDR5 at 5600 MHz or higher with not less than two physical SO-DIMM slots; ot less than 1 TB PCIe Gen4 ×4 NVMe M.2 SSD, installed in an M.2 slot, with at least two internal M.2 slots, of which at least one slot shall be free (unoccupied) at delivery; display not less than 16.0-inch anti-glare, 16:10 aspect ratio; resolution not less than WUXGA (1920 × 1200); color coverage not less than 100% sRGB; brightness not less than 300 nits; ports not less than 2 × Thunderbolt 4, Thunderbolt 5, or USB4 40 Gbps with Power Delivery and DisplayPort Alt Mode, not less than 2 × USB Type-A (5 Gbps or higher), not less than 1 × HDMI 2.1, and not less than 1 × integrated RJ-45 Gigabit Ethernet port; wireless connectivity Wi-Fi 6E or higher and Bluetooth 5.3 or newer; integrated TPM 2.0; integrated camera not less than 720p with physical privacy shutter; full-size backlit spill-resistant QWERTY keyboard; operating system Windows 11 Pro 64-bit OEM Business or Retail license; battery capacity not less than 85 Wh with fast-charge support; weight not more than 3.0 kg. |  |  |  |
| 1.3.4 | Professional 16-inch Ultra-Performance Mobile Workstation (Qwerty EN) Processor not less than Intel Core i7 or i9 HX-series, Intel Core Ultra 7 or 9, or AMD Ryzen 7 or 9 class; minimum 20 logical threads or equivalent; graphics adapter with not less than 12 GB GDDR6 VRAM and support for ISV-certified applications; memory not less than 64 GB DDR5 at 5600 MHz or higher with not less than four physical SO-DIMM slots; storage not less than 2 TB PCIe Gen4 ×4 NVMe M.2 SSD with not less than two internal M.2 SSD slots supporting RAID 0/1; display not less than 16.0-inch anti-glare, 16:10 aspect ratio; resolution not less than WQXGA (2560 × 1600); color coverage not less than 100% sRGB; brightness not less than 400 nits; ports not less than 2 × Thunderbolt 4 or USB4 40 Gbps with Power Delivery and DisplayPort Alt Mode, not less than 2 × USB Type-A (5 Gbps), not less than 1 × HDMI 2.1, and not less than 1 × integrated RJ-45 Ethernet port; wireless connectivity Wi-Fi 6E or Wi-Fi 7 and Bluetooth 5.3 or higher; operating system Windows 11 Pro 64-bit or newer, OEM Business or Retail license; hardware TPM 2.0; fingerprint sensor; physical webcam privacy shutter; backlit spill-resistant QWERTY keyboard; battery capacity not less than 90 Wh with fast-charge support; weight not more than 3.0 kg. |  |  |  |
| 1.3.5 | Laptop – Standard 15/16” (Qwerty EN) Display not less than 15-inch Full HD; processor not less than Intel Core i7 or AMD Ryzen 7; memory not less than 32 GB RAM; storage not less than 1 TB SSD; integrated Gigabit Ethernet or manufacturer-supported adapter; USB-C Power Delivery; wireless connectivity Wi-Fi 6; TPM 2.0; operating system Windows Pro OEM Business or Retail license. |  |  |  |
| 1.3.6 | Laptop – Advanced Workstation (Qwerty EN) Display not less than 17-inch Full HD; Processor x86-64 architecture, Intel Core i9 (or latest generation) or AMD Ryzen 9 (minimum 7000 series or newer), or equivalent; memory not less than 64 GB RAM; graphics adapter with not less than 8 GB dedicated VRAM; storage not less than 1 TB SSD installed, with support for dual internal SSD configuration and total capacity expandable to not less than 2 TB; integrated Gigabit Ethernet or manufacturer-supported adapter; USB-C Power Delivery; wireless connectivity Wi-Fi 6; TPM 2.0; operating system Microsoft Windows 10 Pro or Windows 11 Pro, OEM Business or Retail license. |  |  |  |
| 1.3.7 | Laptop – Professional class (Qwerty EN) Laptop; machine with operating system macOS pre-installed to ensure compatibility with existing security-level environment; processor based on ARM architecture with integrated CPU and GPU in a unified system-on-chip design; unified memory not less than 24 GB; storage not less than 1 TB SSD. |  |  |  |
| 1.3.8 | Laptop – High-End Class (Qwerty EN) Laptop; operating system macOS pre-installed to ensure compatibility with existing security-level environment; processor based on ARM architecture with integrated CPU and GPU in a unified system-on-chip design; unified memory not less than 48 GB; storage not less than 2 TB SSD. |  |  |  |
| 1.4 | LAPTOP ACCESSORIES |  |  |  |
| 1.4.1 | Laptop bag 15" Laptop size supported not less than 15-inch; horizontal messenger bag configuration; padded carry handle; zipper closure; not less than 1 laptop compartment, not less than 1 additional full-size or cable-management compartment, and not less than 1 organizer pocket; weather-resistant material; delicate colour. |  |  |  |
| 1.4.2 | Laptop bag 17" Laptop size supported not less than 17-inch; horizontal messenger bag configuration; padded carry handle; zipper closure; not less than 1 laptop compartment, not less than 1 additional full-size or cable-management compartment, and not less than 1 organizer pocket; weather-resistant material; delicate colour. |  |  |  |
| 1.4.3 | Multi-configuration professional laptop and camera backpack 15" Internal volume not less than 26 liters; internal divider system capable of housing not less than 2 mirrorless or DSLR bodies, including at least 1 body with 70–200 mm f/2.8 attached, and not less than 4 lenses; dedicated padded compartment for laptop not less than 15.6-inch; top, front, and dual-side access; carry options supporting backpack, sling, or duffle configuration; water-repellent ripstop material; lockable zippers and polymer buckles; dual-sided weather cover; airline carry-on compliant dimensions; weight not more than 2.5 kg unloaded; external tripod or monopod attachment points. |  |  |  |
| 1.4.4 | On-Ear Headband USB Headset USB-C and USB-A headset with microphone noise-cancelling technology; microphone arm; call controls; compatibility with Windows and Mac OS; certification for Microsoft Teams and Zoom or equivalent; user hearing protection compliant with EU Noise at Work, G616, or equivalent or superior; headset weight not more than 200 g; busylight and incoming-call indicators; speaker size not less than 40 mm; speaker max input power not less than 30 mW; speaker frequency range 20 Hz to 20,000 Hz; microphone type MEMS or equivalent; operating temperature from -10 °C to +55 °C; box contents including headset, carry pouch, and user documentation. |  |  |  |
| 1.4.5. | High-capacity, high-output Portable Power Bank Capacity not less than 25,000 mAh (90 Wh); total max output not less than 165 W; USB-C output per port not less than 100 W; USB-A output not less than 20 W; ports not less than 2 × USB-C and 1 × USB-A; weight not more than 700 g; digital display; pass-through charging. |  |  |  |
| 1.4.6 | Thunderbolt Docking Station for Laptop, Interface not less than Thunderbolt 4 (40 Gbps) or USB4; backward compatible with USB-C DisplayPort Alt Mode and USB 3.2; Power Delivery not less than 100 W; support for not less than two external displays; fully compatible with items 1.3.1 to 1.3.8. |  |  |  |
| 1.5 | INPUT DEVICES |  |  |  |
| 1.5.1 | Mouse, Optical (Wired) USB-A interface; optical sensor; cable length not less than 1 m; standard desktop size suitable for both hands; not less than 3 buttons including scroll wheel; adjustable DPI including 1600 dpi; compatible with Microsoft Windows. |  |  |  |
| 1.5.2 | Ambidextrous Mouse, Optical (Wireless) Ambidextrous design; LED optical sensor; wireless connectivity via RF 2.4 GHz and Bluetooth 5.0; operating distance not less than 10 m; adjustable DPI including 1000, 1600, and 2400; not less than 3 buttons including scroll wheel; battery-powered using AA or AAA batteries, included; compatible with Windows 10/11 and macOS. |  |  |  |
| 1.5.3 | Keyboard, Wired (Qwerty EN) Wired USB-A interface; cable length not less than 1.5 m; keyboard layout QWERTY US ISO or English International ANSI; compatible with Microsoft Windows operating systems; full-size desktop keyboard; adjustable height with two positions; integrated numeric keypad. |  |  |  |
| 1.5.4 | Keyboard and mouse set, Wireless, (Qwerty EN) Full-size wireless keyboard with numeric keypad; QWERTY layout with function keys F1–F12; membrane or dome-style keys; wireless operation via 2.4 GHz shared USB dongle; operating range not less than 10 m; keyboard powered by 2 × AAA batteries included; spill-resistant design; adjustable tilt feet; Caps Lock and Num Lock indicators; wireless optical mouse with not less than 1000, 1600, and 2400 DPI settings, left and right buttons, scroll wheel, and DPI switch; mouse powered by 1 × AA battery included; ambidextrous design; 1 × nano USB receiver included; plug-and-play; compatible with Windows 10/11 and macOS; black matte finish. |  |  |  |
| 1.5.5 | Wireless presenter with built-in laser pointer Dual connectivity via 2.4 GHz wireless with USB-A receiver or Bluetooth Low Energy; operating range not less than 15 m line of sight; Class 1 red laser pointer; battery type 1 × AAA included; plug-and-play; compatible with Windows 10/11 or later and macOS 10.12 or later; controls including on/off, slide forward/backward, start/exit slideshow, screen blank/resume, and laser activation; LED power indicator; integrated storage for USB receiver. |  |  |  |
| 1.6 | PORTABLE MEDIA DEVICES All storage media devices should be sourced from manufacturers with proven market presence and support infrastructure for after sale services(warranty minimum 12 months or manufacturer’s standard warranty if longer, fully valid and enforceable in Ukraine) and should be supporting wear-leveling and SMART monitoring where applicable). |  |  |  |
| 1.6.1 | External, Multi-format Card Reader Connectivity via USB 3.2 Gen 1 (5 Gbps) Type-A male connector using Micro USB 3.0 to USB Type-A cable included; data transfer rate not less than 5 Gbps; backward compatible with USB 2.0 and USB 1.1; hot-swap and plug-and-play support; LED power and access indicator; compatibility with Microsoft Windows and macOS according to features and compatibility characteristics equivalent to “Delock 91704 All-in-One Card Reader” for operability with existing equipment already possessed by the consignee. |  |  |  |
| 1.6.2 | Micro SD Card, 32GB Storage capacity not less than 32 GB; speed class not less than UHS-I / U3; card reader adapter included. |  |  |  |
| 1.6.3 | Micro SD Card, 64GB Storage capacity not less than 64 GB; speed class not less than UHS-I / U3; card reader adapter included. |  |  |  |
| 1.6.4 | Micro SD Card, 128GB Storage capacity not less than 128 GB; speed class not less than UHS-I / U3; card reader adapter included. |  |  |  |
| 1.6.5 | Micro SD Card, 256GB Storage capacity not less than 256 GB; speed class not less than UHS-I / U3. |  |  |  |
| 1.6.6 | Micro SD Card, 256GB Storage capacity not less than 256 GB; speed class not less than V30. |  |  |  |
| 1.6.7 | USB 3.2 Gen 1 Flash Drive, 128GB USB 3.2 Gen 1 Type-A interface; storage capacity not less than 128 GB; read speed not less than 200 MB/s; write speed not less than 60 MB/s; operating temperature 0 °C to 45 °C; compatible with Windows 10/11 and Mac OS 10.10 or later. |  |  |  |
| 1.6.8 | USB 3.2 Gen 1 Flash Drive, 512GB USB 3.2 Gen 1 Type-A interface; storage capacity not less than 512 GB; read speed not less than 130 MB/s; write speed not less than 30 MB/s; operating temperature 0 °C to 45 °C; compatible with Windows 10/11 and Mac OS 10.10 or later. |  |  |  |
| 1.6.9 | Solid State Flash Drive, 1 TB USB 3.2 Gen 2 interface, backward compatible with USB 3.0 and USB 2.0; storage capacity not less than 1 TB; read speed not less than 420 MB/s; write speed not less than 380 MB/s; operating temperature 0 °C to 35 °C; compatible with Windows 10/11 and Mac OS 10.10 or later. |  |  |  |
| 1.7 | CABLES, AUDIO/VIDEO |  |  |  |
| 1.7.1 | DisplayPort cable, male-to-male, length not less than 5 m) DisplayPort cable; male-to-male; length not less than 5 m. |  |  |  |
| 1.7.2 | 1m High Speed HDMI Cable 2 × HDMI Type-A male connectors; support for not less than 4K at 30 Hz; length not less than 1 m. |  |  |  |
| 1.7.3 | 5m High Speed HDMI Cable 2 × HDMI Type-A male connectors; support for not less than 4K at 30 Hz; length not less than 5 m. |  |  |  |
| 1.7.4 | 10m High Speed HDMI Cable 2 × HDMI Type-A male connectors; support for not less than 4K at 30 Hz; length not less than 10 m. |  |  |  |
| 1.7.5 | 30m High Speed HDMI Cable 2 × HDMI Type-A male connectors; support for not less than 4K at 30 Hz; length not less than 30 m. |  |  |  |
| 1.7.6 | High performance HDMI Cable 5m golden connectors HDMI v2.0 male-to-male cable; support for not less than 4K at 60 Hz; gold-plated connectors; length not less than 5 m. |  |  |  |
| 1.7.7 | High performance HDMI Cable 10m golden connectors HDMI v2.0 male-to-male cable; support for not less than 4K at 60 Hz; gold-plated connectors; length not less than 10 m. |  |  |  |
| 1.7.8 | Audio equipment connection Cable XLR female to XLR male cable; halogen-free; length not less than 20 m. |  |  |  |
| 1.8 | POWER STRIPS/ POWER EXTENSION CABLES |  |  |  |
| 1.8.1 | Power Strip (type 1) Power strip; voltage 220–240 V AC, 50 Hz; rated current not less than 16 A; rated power not less than 3500 W; equipped with not less than 6 outlets of type CEE 7/4 (Schuko) or equivalent; cable length not less than 3 m; plug type CEE 7/7 (Schuko) or equivalent; suitable for indoor use; CE compliant. |  |  |  |
| 1.8.2 | Power Strip (type 2) Power strip; voltage 220–240 V AC, 50 Hz; rated current not less than 16 A; rated power not less than 3500 W; equipped with not less than 4 outlets of type CEE 7/4 (Schuko) or equivalent; cable length not less than 2 m; plug type CEE 7/7 (Schuko) or equivalent; suitable for indoor use; CE compliant. |  |  |  |
| 1.8.3 | Power Strip (type 3) Power strip; voltage 220–240 V AC, 50 Hz; rated current not less than 16 A; rated power not less than 3500 W; equipped with not less than 8 outlets of type CEE 7/4 (Schuko) or equivalent; cable length not less than 5 m; plug type CEE 7/7 (Schuko) or equivalent; suitable for indoor use; CE compliant. |  |  |  |
| 1.8.4 | Power Cable 5m Power extension cable; voltage 220–240 V; current not less than 16 A; cable 3 × 1.5 mm²; CEE 7/7 male to CEE 7/4 female; length not less than 5 m; indoor usage; CE compliant. |  |  |  |
| 1.8.5 | Power Cable 10m Power extension cable; voltage 220–240 V; current not less than 16 A; cable 3 × 1.5 mm²; CEE 7/7 male to CEE 7/4 female; length not less than 10 m; indoor usage; CE compliant. |  |  |  |
| 2 | UNINTERRUPTIBLE POWER SUPPLY |  |  |  |
| 2.1 | DESKTOP PROTECTION |  |  |  |
| 2.1.1 | Uninterrupted Power Supply 950VA Capacity 900–1000 VA; output 230 V AC; USB connectivity; European type plug; grounded; not less than 4 Schuko AC sockets; battery backup time not less than 5 minutes at 50% load; F-type charging/supply cable length not less than 1 m. |  |  |  |
| 2.1.2 | OEM-compatible Replacement Battery Cartridge Fully compatible with item 2.1.1; drop-in replacement compatibility. |  |  |  |
| 2.1.3 | Uninterruptible Power Supply Capacity not less than 1500 VA; output 230 V AC; USB connectivity; European type plug; grounded; not less than 6 Schuko AC sockets; battery backup time not less than 5 minutes at 50% load; F-type charging/supply cable length not less than 1 m. |  |  |  |
| 2.1.4 | OEM-compatible Replacement Battery Cartridge Fully compatible with item 2.1.3; drop-in replacement compatibility. |  |  |  |
| 2.1.5 | OEM-compatible Replacement Battery Cartridge Fully compatible with APC SMT1000C for ensuring compatibility and operability with existing infrasfructures already possessed by the consignee; drop-in replacement compatibility. |  |  |  |
| 2.2 | RACK-MOUNTABLE |  |  |  |
| 2.2.1 | Uninterruptible Power Supply, 3000VA Capacity not less than 3 kVA; rack height not more than 2U; double-conversion online; bypass mode; RJ-45 and USB connectivity; European grounded plug; not less than 6 IEC and C19 sockets. |  |  |  |
| 2.2.2 | Uninterruptible Power Supply, 6000VA Capacity not less than 6 kVA; rack height not more than 4U; double-conversion online; bypass mode; RJ-45 and USB connectivity; European grounded plug; not less than 8 IEC and C19 sockets. |  |  |  |
| 2.2.3 | OEM-compatible uninterruptible Power Supply Battery Extension Pack 5 kVA to UPS 6 kVA Fully compatible with item 2.2.2; drop-in replacement compatibility; rack height not more than 3U; weight not more than 100 kg. |  |  |  |
| 2.3 | PORTABLE POWER STATIONS |  |  |  |
| 2.3.1 | Standard Portable Power station Battery capacity not less than 1 kWh; battery chemistry LFP; AC output capacity not less than 1500 W with surge not less than 2000 W; AC charging time from 0% to 100% not more than 2 hours; ports not less than 2 × USB-C at 100 W each, not less than 2 × USB-A, and not less than 2 × Schuko AC sockets; certifications including CE, FCC, RoHS, UN38.3, and UL. |  |  |  |
| 2.3.2 | Advanced Portable Power station Battery capacity not less than 4 kWh; battery chemistry LFP; AC output capacity not less than 4000 W with surge not less than 6000 W; AC charging time from 0% to 100% not more than 4 hours; ports not less than 2 × USB-C at 100 W each, not less than 2 × USB-A, and not less than 2 × Schuko AC sockets; support for additional battery modules; certifications including CE, FCC, RoHS, UN38.3, and UL. |  |  |  |
| 3 | DISPLAY AND PRESENTATION DEVICES |  |  |  |
| 3.1 | DESKTOP DISPLAYS |  |  |  |
| 3.1.1 | Desktop LCD Display, Basic (24", Full HD) Display diagonal not less than 24-inch; resolution not less than Full HD; IPS panel; ports including HDMI and DisplayPort; native aspect ratio 16:9; non-touch; brightness min 250 nits; response time max. 5 ms; contrast ratio not less than 1000:1; scanning frequency 48–76 Hz vertical and 24–94 kHz horizontal. |  |  |  |
| 3.1.2 | 27" (or higher) 4K UHD Professional USB-C Monitor Display size not less than 27-inch; IPS or equivalent panel; native resolution not less than 3840 × 2160; refresh rate not less than 120 Hz; Adaptive-Sync support; static contrast ratio not less than 1500:1; color gamut not less than 99% sRGB; brightness not less than 350 cd/m²; USB-C or equivalent supporting DisplayPort Alt Mode, data, and Power Delivery not less than 65 W; video inputs not less than 2 × HDMI 2.1 and 1 × DisplayPort 1.4; integrated USB hub with not less than 2 × USB-A ports; integrated stereo speakers not less than 2 × 5 W; fully adjustable stand with height adjustment not less than 100 mm, tilt, swivel, and pivot; VESA 100 × 100 mm; hardware low blue light and flicker-free technology; AC 100–240 V, 50/60 Hz; detachable Schuko to IEC C13 cable not less than 1.5 m or manufacturer’s standard length. |  |  |  |
| 3.1.3 | Professional 31.5" 4K Thunderbolt 4 Monitor Display size 31.5-inch; IPS Black or equivalent panel; resolution not less than 3840 × 2160 at 60 Hz; aspect ratio 16:9; static contrast ratio not less than 2000:1; brightness not less than 400 nits with DisplayHDR 400 support; color coverage not less than 98% Display P3 and 100% sRGB, factory calibrated with Delta E lower than 2; Thunderbolt 4 or USB4 primary port with Power Delivery not less than 100 W; not less than 1 × DisplayPort 1.4, not less than 1 × HDMI 2.0, integrated USB 3.2 Gen 2 hub with not less than 4 × USB-A and 1 × downstream USB-C; integrated RJ-45 Gigabit Ethernet; integrated hardware KVM switch; fully adjustable stand with height adjustment not less than 150 mm, tilt, swivel, and pivot; flicker-free and hardware low blue light filter; internal power supply; detachable Schuko to IEC C13 cable not less than 1.5 m or manufacturer’s standard length. |  |  |  |
| 3.1.4 | 34" Ultrawide USB-C Productivity Monitor Display size not less than 34-inch; ultrawide curved panel of not less than 1800R; resolution not less than 3440 × 1440; aspect ratio 21:9; VA or equivalent panel with native contrast ratio not less than 4000:1; refresh rate not less than 100 Hz; USB-C or equivalent supporting DisplayPort Alt Mode, data, and Power Delivery not less than 65 W; integrated KVM functionality; not less than 1 × HDMI 2.1 and 1 × DisplayPort 1.4; integrated USB 3.2 Gen 1 hub with not less than 2 × USB-A; integrated stereo speakers not less than 2 × 5 W; height-adjustable stand not less than 100 mm and tilt support; VESA 100 × 100 mm; flicker-free and hardware low blue light filter; AC 100–240 V with detachable Schuko to IEC C13 cable not less than 1.5 m or manufacturer’s standard length. |  |  |  |
| 3.2 | WALL-MOUNTED DISPLAYS AND ACCESSORIES |  |  |  |
| 3.2.1 | 75" Professional Display Diagonal size not less than 75-inch; signage grade for 24/7 operation; panel refresh rate not less than 60 Hz; resolution not less than 4K UHD (3840 × 2160); brightness not less than 350 cd/m²; contrast ratio not less than 4000:1; viewing angles not less than 178° horizontal and vertical; inputs not less than 3 × HDMI and 1 × DisplayPort; data ports not less than 2 × USB and 1 × LAN (RJ45); RS-232C and IR external control; integrated Wi-Fi and Bluetooth; embedded operating system with integrated media player; landscape and portrait support; VESA 400 × 400 mm with M8 compatibility; weight not more than 40 kg; AC 100–240 V, 50/60 Hz; detachable Schuko to IEC C13 grounded cable not less than 1.5 m or manufacturer’s standard length. |  |  |  |
| 3.2.2 | Wall Mount, f/ Wall Display VESA compatibility 400 × 400 mm; weight capacity not less than 50 kg; fixed mount type; M8 screws included; heavy-duty cold-rolled steel; low-profile wall clearance 20–40 mm; compatible with item 3.2.1. |  |  |  |
| 3.3 | PROJECTORS AND ACCESSORIES |  |  |  |
| 3.3.1 | Portable LED Projector LED light source with life not less than 30,000 hours; brightness not less than 300 LED lumens or 120 ANSI lumens; native resolution not less than 854 × 480, supporting input up to 1920 × 1080; throw ratio not more than 1.2:1; integrated rechargeable battery with video playback not less than 2 hours in standard mode; built-in stereo speakers with total output not less than 5 W; ports not less than 1 × HDMI v1.4 or higher and 1 × USB-C; built-in Wi-Fi and Bluetooth; weight not more than 1 kg. |  |  |  |
| 3.3.2 | Stationary LED Projector LED light source with life not less than 30,000 hours; brightness not less than 3000 ANSI lumens; native resolution not less than 1280 × 800, supporting input up to Full HD or WUXGA; aspect ratio 16:10; contrast ratio not less than 3,000,000:1; not less than 1 × HDMI v1.4 or higher; AC 100–240 V, 50/60 Hz; detachable Schuko to IEC C13 grounded cable not less than 1.5 m or manufacturer’s standard length; weight not more than 3.0 kg. |  |  |  |
| 3.3.3 | Portable Tripod Projection Screen Portable tripod projection screen; manual pull-up screen with integrated folding tripod stand; durable metal casing with carrying handle and transport lock; screen size not less than 85-inch class; viewing area not less than 170 × 170 cm or equivalent; support for multiple aspect ratios including 1:1, 4:3 and 16:9 via adjustable height mechanism; matte white projection surface with black opaque backing; gain between 1.0 and 1.2; viewing angle not less than 160°; black borders not less than 2.5 cm on all sides; flame-retardant and mildew-resistant material; stable base suitable for indoor use; foldable and transportable design; compatible with standard portable and stationary projectors; warranty minimum 12 months. |  |  |  |
| 4 | NETWORKING |  |  |  |
| 4.1 | SWITCHES |  |  |  |
| 4.1.1 | Network Switch 8-ports PoE Unmanaged Gigabit Ethernet switch; not less than 8 PoE+ ports; total PoE budget not less than 120 W; ports supporting 10/100/1000 Mbps. To be compatible with requirements for NETGEAR GS108PP-100EUS for operability with equipment already possessed by the consignee. |  |  |  |
| 4.1.2 | Multi-Gigabit Managed Ethernet Switch Total ports not fewer than 10; not fewer than 2 × RJ-45 ports supporting 2.5G/5G/10G; not fewer than 2 × SFP+ slots supporting 10 Gbps; not fewer than 8 × RJ-45 ports supporting 10/100/1000 Mbps; switching capacity not less than 60 Gbps; forwarding rate not less than 44 Mpps; MAC address table not fewer than 16K entries; desktop or rack-mountable metal or composite housing; 19-inch rack kit or brackets included; passive or smart-fan low-noise cooling; support for IEEE 802.3ab, 802.3bz, 802.3ae/an, and 802.3az. |  |  |  |
| 4.1.3 | Network Switch 24- ports PoE Not less than 24 full PoE+ ports, including 16 × 1 GbE RJ45 and 8 × mGbE RJ45 supporting 100M/1G/2.5G/5G/10G; not less than 4 × 10GbE SFP+ uplinks; not less than 2 × 40G hardware stack ports; stacking bandwidth not less than 160 Gbps; switching capacity not less than 176 Gbps; Layer 3 routing; dual hot-swap power supply; hot-swap fans. To be compatible with requirements for, and features of, Cisco Catalyst 9300L/9300 Series for operability with equipment already possessed by the consignee. |  |  |  |
| 4.1.4 | Network Switch 48- ports PoE Not less than 48 full PoE+ ports; not less than 4 × 10GbE SFP+ uplinks; not less than 2 × 40G hardware stack ports; stacking bandwidth not less than 160 Gbps; switching capacity not less than 176 Gbps; Layer 3 routing; dual hot-swap power supply; hot-swap fans. To be compatible with requirements for, and features of, Cisco Catalyst 9300L/9300 Series, for operability with equipment already possessed by the consignee. |  |  |  |
| 4.1.5 | Network Switch, 24 ports with Layer2/3 features Managed Gigabit switch; not less than 24 × 10/100/1000 RJ-45 PoE+ ports; not less than 4 × 1 Gb SFP uplinks; flash not less than 16 GB; DRAM not less than 8 GB; switching capacity not less than 100 Gbps; PoE/PoE+ IEEE 802.3af/at; support for VLAN, ACL, SNMP, 802.1Q, and 802.1X; internal dual or redundant power supply; 1RU rack-mountable; input voltage 230 V AC. To be compatible with requirements for, and features of, Cisco Catalyst 3850 Series or equivalent, for operability with equipment already possessed by the consignee. |  |  |  |
| 4.2 | ROUTING & FIREWALL |  |  |  |
| 4.2.1 | High-Performance Aggregation Router 19-inch rack-mountable chassis not more than 2U; memory not less than 16 GB RAM and 32 GB internal storage; forwarding capacity not less than 40 Gbps aggregate non-blocking throughput with services enabled; IPsec encryption throughput not less than 15 Gbps with hardware acceleration; dedicated hardware packet processing engine; not less than 4 × 10/40/100 GbE multi-rate ports; not less than 8 × 1/10 GbE SFP/SFP+ ports; dedicated management and console access; native SD-WAN and centralized orchestration support; full support for BGP with not less than 1M IPv4 routes, OSPF, IS-IS, MPLS L2/L3 VPN; IPv4/IPv6 dual stack; multicast PIM-SM/SSM; Segment Routing; MACsec and Zone-Based Firewall support; NETCONF, RESTCONF, and YANG model support to ensure full operability with compatible equipoment already possessed by the consignee; dual hot-swappable AC/DC power supplies (N+1). |  |  |  |
| 4.2.2 | High-Performance Enterprise Core Router 19-inch rack-mountable chassis 1U; active cooling with redundant fans; 64-bit multi-core CPU with not less than 16 physical cores; memory not less than 16 GB RAM; hardware Layer 3 forwarding and IPsec encryption; not less than 4 × 10G SFP+ ports; not less than 12 × 10/100/1000 Mbps RJ-45 ports; dedicated console and management ports; full support for IPv4/IPv6 dual stack, OSPF v2/v3, and BGP with not less than 1M IPv4 routes; support for MPLS, VRF, and Policy-Based Routing; hardware-accelerated support for IPsec, OpenVPN, and WireGuard; dual hot-swappable AC power supplies (N+1); SNMP v2/v3, SSH, HTTPS, and NETCONF/RESTCONF or equivalent API. |  |  |  |
| 4.2.3 | Industrial Router 3/4G with accessories Industrial cellular router; device shall support not less than 4 Ethernet ports including minimum 1 WAN and 3 LAN Gigabit Ethernet interfaces; integrated cellular modem supporting 3G/4G LTE (Category 6 or higher); dual SIM functionality with automatic failover; dual-band Wi-Fi compliant with IEEE 802.11ac or equivalent; the device shall be designed for industrial use including operating temperature range not less than –40 °C to +75 °C, ruggedized enclosure suitable for harsh environments, wide power input range (not less than 9–50 V DC), and support for continuous operation in unstable power conditions; the device shall include external antennas (minimum 2 for cellular and 2 for Wi-Fi), USB interface (Type-A or equivalent), support for remote management and configuration (web interface and/or centralized management platform), and support for VPN protocols (IPSec, OpenVPN or equivalent), firewall, and network failover/load balancing; the solution shall be fully compatible with standard networking infrastructure and existing equipment already possessed by the consignee; accessories shall include AC power supply and DC power cable; warranty minimum 12 months. |  |  |  |
| 4.2.4 | Mobile Data Router Mobile data router; cellular connectivity supporting LTE Category 16 or higher and/or 5G equivalent; support for not less than 20 simultaneously connected devices; dual-band Wi-Fi (2.4 GHz and 5 GHz) compliant with IEEE 802.11ac or equivalent; integrated SIM slot (single or dual SIM) with support for automatic network selection; external LTE/5G antenna connectors; at least one Ethernet port (Gigabit preferred); integrated rechargeable battery with autonomy not less than 8 hours under normal operating conditions; compact and portable design with weight not exceeding 0.5 kg; support for web-based management interface and basic network security features (NAT, firewall or equivalent); USB interface (Type-C or equivalent) for charging and/or data connectivity; compatibility with major European frequency bands; accessories shall include AC charger and USB-C cable; warranty minimum 12 months. |  |  |  |
| 4.2.5 | Gigabit Multi-WAN VPN Router Not less than 5 × 10/100/1000 Mbps RJ-45 ports; not less than 3 changeable WAN/LAN ports; concurrent sessions not less than 25,000; IPsec VPN throughput not less than 40 Mbps; support for IPsec, PPTP, L2TP, and OpenVPN; support for not less than 20 IPsec tunnels and not less than 15 L2TP/PPTP tunnels; SPI firewall; IP/MAC/URL filtering; 802.1Q VLAN; web-based GUI over HTTP/HTTPS; centralized or cloud management support; fanless cooling; metal or composite housing. |  |  |  |
| 4.2.6 | Firewall Rack-mounted network multifunctional security and SD-WAN appliance; Layer 7 firewall with identity-based policies and application management; malware protection with sandboxing and file reputation; intrusion prevention with PCI-compliant IPS; category-based content filtering; SSL inspection; DLP; CASB; SaaS tenant restrictions; application and file type control; built-in DHCP, NAT, QoS, and VLAN services; web caching; load balancing and failover; automatic detection of Layer 2 and Layer 3 outages; NGFW throughput not less than 2.5 Gbps; site-to-site VPN throughput not less than 2.5 Gbps; not less than 500 site-to-site VPN tunnels; integrated LAN interfaces not less than 4 × 1GbE and 2 × 10GbE SFP+; integrated WAN interfaces not less than 2 × 10GbE SFP+; AC input voltage 200–240 VAC; AC frequency 50/60 Hz. |  |  |  |
| 4.2.7 | Enterprise-level Next-Generation Firewall 19-inch rack-mountable chassis 1U; internal storage not less than 120 GB SSD/NVMe; firewall throughput not less than 18 Gbps (UDP 1518B); IPS throughput not less than 2.5 Gbps; NGFW / threat protection throughput not less than 1 Gbps with IPS, App Control, and Malware Protection enabled; SSL inspection not less than 1 Gbps; concurrent sessions not less than 1.5 million TCP; not less than 2 × 10G SFP+ uplinks; not less than 8 × 10/100/1000 Mbps RJ-45 ports; not less than 8 × 1G SFP slots, including shared/combo if applicable; dedicated management and console ports; IPsec VPN throughput not less than 10 Gbps and SSL VPN support for not less than 500 concurrent users; integrated dual redundant power supplies; AC 100–240 V; compatible with Schuko sockets. |  |  |  |
| 4.3 | WIRELESS CONNECTIVITY |  |  |  |
| 4.3.1 | Access Point Dual-band Wi-Fi 6 access point with 2×2:2 MU-MIMO on 5 GHz and 2.4 GHz; aggregate throughput not less than 1.5 Gbps; integrated enterprise security and guest access; application-aware traffic shaping; self-configuring deployment; internal antenna gain not less than 5.4 dBi at 2.4 GHz and 6 dBi at 5 GHz; power consumption not more than 15 W under 802.3af; not less than 1 × 10/100/1000 RJ45 Ethernet port; integrated Layer 7 firewall; WIDS/WIPS and rogue AP detection; guest isolation; VLAN tagging and IPsec tunneling; PCI compliance reporting; support for WEP, WPA, WPA2-PSK, WPA2-Enterprise with 802.1X, WPA3-Personal, WPA3-Enterprise, and OWE; support for EAP-TLS, EAP-TTLS, EAP-MSCHAPv2, and EAP-SIM; TKIP and AES; EMM and MDM integration. |  |  |  |
| 4.3.2 | High-Gain Wi-Fi 6 Access Point Concurrent dual-band Wi-Fi 6 (802.11ax); aggregate throughput not less than 1750 Mbps; integrated antenna gain not less than 6 dBi at 2.4 GHz and 5.5 dBi at 5 GHz; not less than 2×2 MU-MIMO on both bands; not less than 2 × 10/100/1000 RJ-45 ports; PoE-Out support on secondary port; support for 802.3af/at PoE and passive PoE 18–57 V; support for AP, router, and mesh modes; standalone and centralized controller management; WPA3-Personal and WPA3-Enterprise; universal bracket included; operating temperature from -40 °C to +70 °C. |  |  |  |
| 4.4 | FLAT PATCH CORD |  |  |  |
| 4.4.1 | UTP Cat6 flat patch cord, not less than 1 m |  |  |  |
| 4.4.2 | UTP Cat6 flat patch cord, not less than 2 m |  |  |  |
| 4.4.3 | UTP Cat6 flat patch cord, not less than 3 m |  |  |  |
| 4.4.4 | UTP Cat6 flat patch cord, not less than 5 m |  |  |  |
| 4.4.5 | UTP Cat6 flat patch cord, not less than 10 m |  |  |  |
| 4.4.6 | UTP Cat6 flat patch cord, not less than 20 m |  |  |  |
| 4.5 | NETWORK INFRASTRUCTURE INSTALLATION |  |  |  |
| 4.5.1 | CAT6a FTP indoor installation cable CAT6a FTP indoor installation cable; twisted pair copper cable compliant with Category 6a standards (ISO/IEC 11801 or equivalent or superior); support for data transmission up to 10 Gbps; shielding type FTP or equivalent; suitable for structured cabling installations; supplied in bulk length not less than 250 m per reel or box; conductor material solid copper; compatible with PoE/PoE+ applications; outer sheath suitable for indoor installation with low smoke and flame-retardant properties (LSZH or equivalent where required); compatible with standard RJ45 termination systems; warranty minimum 12 months. |  |  |  |
| 4.5.2 | CAT6a connectors (100pcs/ pack) CAT6a connectors (100 pcs per pack); RJ45 connectors compatible with Category 6a cabling (FTP/STP as applicable); suitable for termination of solid or stranded copper conductors; compliant with ISO/IEC 11801 or equivalent or superior standards; support for data transmission up to 10 Gbps or superior; compatible with standard crimping tools and network installation practices; shielding where applicable; quantity not less than 100 pieces per pack; warranty minimum 12 months. |  |  |  |
| 4.5.3 | USB 3.0 (USB 3.2 Gen 1) Male to Female extension cable. USB 3.0 / USB 3.2 Gen 1 male-to-female extension cable; backward compatible with USB 2.0 and USB 1.1; length not more than 3 m. |  |  |  |
| 5 | PRINTING & SCANNING |  |  |  |
| 5.1 | MULTI-FUNCTION PRINTER & COPIERS |  |  |  |
| 5.1.1 | Standard Monochrome Multifunctional Printer (MFP) Functions including print, copy, color scan, and digital send; print speed not less than 30 ppm A4; resolution not less than 600 × 600 dpi; memory not less than 512 MB; automatic duplex printing; manual or automatic simplex scanning; duty cycle not less than 15,000 pages/month; support for high-yield replacement toner cartridges; interfaces including Ethernet (RJ-45), USB 2.0, and Wi-Fi; paper handling with main tray not less than 250 sheets and ADF; media support not less than A4, A5, and A6; AC 220–240 V, 50/60 Hz; detachable Schuko to IEC C13 grounded cable not less than 1.5 m or manufacturer’s standard length. |  |  |  |
| 5.1.2 | Full spare set of high-yield toner cartridge(s) One full spare set of high-yield toner cartridge(s); fully compatible with item 5.1.1. |  |  |  |
| 5.1.3 | Advanced A3 Monochrome Multifunctional Printer (MFP) Functions including print, copy, scan, and digital send; print speed not less than 25 ppm A4; resolution not less than 600 × 600 dpi native and 1200 dpi interpolated; duty cycle not less than 30,000 pages/month; format support including A3, A4, A5, and A6 for print and scan; paper input not less than 250-sheet main tray plus bypass tray; ADF not less than 50 sheets; automatic duplex printing and copying; memory not less than 1 GB; touchscreen not less than 7-inch; Gigabit Ethernet, USB 2.0, and Wi-Fi; support for high-yield toner cartridges; AC 100–240 V, 50/60 Hz; detachable Schuko to IEC C13 grounded cable not less than 1.5 m or manufacturer’s standard length. |  |  |  |
| 5.1.4 | Full spare set of high-yield toner cartridge(s) One full spare set of high-yield cartridge(s); fully compatible with item 5.1.3. |  |  |  |
| 5.1.5 | Color Multifunctional Printer (MFP) Functions including print, copy, scan, and digital send; color laser technology (CMYK); print speed not less than 45 ppm A4 with A3 support; resolution not less than 1200 × 1200 dpi; memory not less than 2 GB; automatic duplex printing, copying, and scanning; duty cycle not less than 20,000 pages/month; support for high-yield toner cartridges; high-speed ADF; media support not less than A3, A4, and A5; interfaces including Gigabit Ethernet and Hi-Speed USB 2.0; integrated color touchscreen not less than 7-inch; AC 100–240 V, 50/60 Hz; detachable Schuko to IEC C13 grounded cable not less than 1.5 m or manufacturer’s standard length. |  |  |  |
| 5.1.6 | Full spare set of high-yield color toner cartridges (CMYK) One full spare set of high-yield color toner cartridges (CMYK); fully compatible with item 5.1.5. |  |  |  |
| 5.2 | INKJET PRINTERS & MULTI-FUNCTION PRINTERS |  |  |  |
| 5.2.1 | Ultra-Portable Single-Function Inkjet Printer Thermal or piezoelectric inkjet printing; weight not more than 2.5 kg; integrated rechargeable battery with charging via AC adapter or USB Type-C; wireless support via Wi-Fi Direct and standard Wi-Fi; USB interface; fold-out top-loading sheet feeder with capacity not less than 10 sheets; LCD display for battery status, ink levels, and network setup; Certified compatibility with AirPrint and Mopria standards, enabling driverless printing from Apple and Android devices over network (Wi-Fi or Ethernet) |  |  |  |
| 5.2.2 | Full spare set of cartridges (B+CMY) One full spare set of cartridges (B+CMY); fully compatible with item 5.2.1. |  |  |  |
| 5.2.3 | Large-Format Multifunctional Printer (MFP) Pigment inkjet system with not less than 5 colors (C, M, Y, BK, Matte BK); printing width up to 36 inches (914 mm) supporting A0, A1, and Arch E; print resolution not less than 2400 × 1200 dpi; single-sensor CIS scanning; optical scan resolution not less than 600 dpi; automatic roll feed with precision cutting; integrated Gigabit Ethernet, Wi-Fi, and high-speed USB; memory/storage not less than 128 GB virtual memory and 500 GB hard drive; acoustic noise level not less than 42 dB. |  |  |  |
| 5.2.4 | Full spare set of high-yield ink cartridges One full spare set of high-yield ink cartridges; fully compatible with item 5.2.3. |  |  |  |
| 5.2.5 | High-Capacity A3 Color Inkjet MFP Thermal or piezoelectric inkjet printing; functions including print, copy, scan, and fax; performance not less than 25 ppm black and 12 ppm color ISO; automatic duplex printing and scanning for A3/A4; paper handling with dual front-loading trays not less than 500 sheets total and ADF not less than 50 sheets; interfaces including Gigabit or Fast Ethernet, Wi-Fi Direct, and Hi-Speed USB; native support for AirPrint and Mopria; integrated color touchscreen not less than 2.7-inch; AC 220–240 V, 50/60 Hz; detachable Schuko to IEC C13 grounded cable not less than 1.5 m or manufacturer’s standard length. |  |  |  |
| 5.2.6 | Full spare set of high-yield ink cartridges (CMYK) One full spare set of high-yield ink cartridges (CMYK); fully compatible with item 5.2.5. |  |  |  |
| 5.3 | SCANNERS |  |  |  |
| 5.3.1 | Flatbed Scanner with ADF Combined flatbed and ADF scanner; ADF speed not less than 50 ppm / 100 ipm A4 at 300 dpi in color or mono duplex; daily duty cycle not less than 8,000 pages; ADF capacity not less than 100 sheets at 80 g/m²; optical resolution not less than 1200 dpi flatbed and 600 dpi ADF; single-pass automatic duplex scanning; high-speed USB 3.0 and Gigabit Ethernet; integrated Wi-Fi 802.11b/g/n with Wi-Fi Direct; support for TWAIN and ISIS; ultrasonic double-feed detection; AC 100–240 V with detachable Schuko to IEC C13 grounded cable not less than 1.5 m or manufacturer’s standard length. |  |  |  |
| 6 | PRESS/CONTENT CREATION EQUIPMENT |  |  |  |
| 6.1 | PHOTO CAMERAS |  |  |  |
| 6.1.1 | Professional-grade full-frame mirrorless camera Professional-grade full-frame mirrorless camera, (body only, without lens), including battery and charger; full-frame CMOS sensor not less than 24 MP; image processor of latest generation or equivalent; ISO range not less than 100–51,200 (expandable); autofocus system with advanced phase-detection covering the majority of the frame; continuous shooting not less than 10 fps mechanical and 20 fps electronic; video not less than 4K 60p 10-bit 4:2:2 and Full HD high frame rate with support for logarithmic video profiles; in-body 5-axis image stabilization; connectivity including Wi-Fi, Bluetooth, USB-C, HDMI, microphone input, and headphone jack; body weight not more than 800 g. |  |  | 15 |
| 6.2 | PHOTO CAMERA ACCESSORIES |  |  |  |
| 6.2.1 | Professional-grade standard zoom lens, 24–70mm Focal length 24–70 mm; constant aperture f/2.8; image stabilization up to 5 stops; autofocus motor Nano USM or equivalent; minimum focus distance 0.21 m at wide and 0.38 m at tele; weather sealing; filter size 82 mm. |  |  |  |
| 6.2.2 | High-performance, professional telephoto zoom lens Focal length 70–200 mm; constant aperture f/2.8; native RF-mount (adapter-less), compatible with Canon RF system cameras already possessed by the consignee; image stabilization up to 5 stops; autofocus system with dual independent ultrasonic motor or equivalent; minimum focus distance not more than 0.7 m; weather sealing; weight not more than 1,070 g. |  |  |  |
| 6.2.3 | Small Rig Cage Kit Aluminum alloy cage kit (fit for Canon EOS R6 Mark II or equivalent to ensure compatibility with existing equipment already possessed by the consignee); mounts including 1/4"-20, 3/8"-16, ARRI locating holes, and cold shoe; full-frame cage with top handle and baseplate for accessories; compatible with item 6.1.1. |  |  |  |
| 6.2.4 | High-Capacity Rechargeable Lithium-Ion Battery Pack Rechargeable lithium-ion battery; nominal voltage 7.2 V DC; rated capacity not less than 2130 mAh; optimized for high-drain imaging devices and 4K/8K recording; integrated microchip for bidirectional communication with host device; USB-C Power Delivery support for in-camera charging; compatible with dedicated cradle chargers of LC-E6 class; protection against overcharge, over-discharge, short-circuit, and overheating; UN38.3, CE, and UL compliant; fully compatible with item 6.1.1. |  |  |  |
| 6.3 | VIDEO CAMERAS |  |  |  |
| 6.3.1 | Semi-Professional 4K Video Camcorder BSI CMOS sensor not less than 1/2.5-inch; native 4K UHD (3840 × 2160) at 25/30 fps or higher; optical zoom not less than 20×; 5-axis hybrid optical image stabilization; wide-angle aperture not slower than f/1.8; dedicated 3.5 mm microphone input and 3.5 mm headphone jack; clean HDMI output Type C or D; integrated Wi-Fi for remote control; flip-out touchscreen not less than 3.0-inch; dedicated manual ring or dial for focus, iris, or zoom; support for SDXC UHS-I Speed Class 3 or higher; support for rechargeable batteries and direct AC power. |  |  |  |
| 6.4 | TABLETS AND ACCESSORIES |  |  |  |
| 6.4.1 | Android-based Tablet, Display not less than 10.9-inch IPS LCD with resolution 1440 × 2304 (WUXGA+) or higher; refresh rate not less than 90 Hz; processor manufactured with no less than 4 nm process technology, octa-core architecture or equivalent and performance suitable for multitasking, video conferencing and office productivity applications; memory not less than 8 GB RAM; internal storage not less than 256 GB with microSDXC expansion up to 1 TB; operating system Android 13 or higher; battery not less than 8,000 mAh Li-Po; connectivity including 5G, Wi-Fi 6, Bluetooth 5.3, GPS, and USB Type-C 2.0; IP68 durability. |  |  |  |
| 6.4.2 | Stylus/Pen (S Pen, or equivalent) IP68-rated stylus or pen; magnetically attachable for charging; fully compatible with item 6.4.1. |  |  |  |
| 6.4.3 | Rugged Enterprise-Grade Tablet Octa-core processor with clock speed not less than 2.4 GHz; display not less than 8.0-inch WUXGA (1920 × 1200) high-brightness panel with high-sensitivity touch for glove and wet-hand operation; memory not less than 6 GB RAM and internal storage not less than 128 GB with microSD expansion; integrated 4G LTE or 5G, Wi-Fi 6, and NFC; user-replaceable battery not less than 5000 mAh with fast charging, No Battery Mode, and industrial POGO pins for cradle charging; IP68-rated active stylus included; USB-C 3.2 Gen 1 for data and video output; MIL-STD-810H and IP68 certification. |  |  |  |
| 6.5 | SOFTWARE |  |  |  |
| 6.5.1 | MS Windows 11 Professional 64-bit (or most current version) Perpetual License Perpetual Retail or ESD license for Windows 11 Professional 64-bit or most current version; supporting BitLocker Drive Encryption, Windows Information Protection, Group Policy, Azure Active Directory Join, Mobile Device Management, Hyper-V, Windows Sandbox, and full Remote Desktop host and client. |  |  |  |
| 6.5.2 | Microsoft Office Home & Business 2024 (or most current version) Perpetual License (Windows OS) Perpetual license for Microsoft Office Home & Business 2024 or most current version for Windows OS; including Microsoft Word, Excel, PowerPoint, Outlook, and OneNote desktop; supported on Windows 10, Windows 11, or most current Windows OS version in 32-bit and 64-bit; language support for all EU languages, multiple language interfaces, and proofing tools. |  |  |  |
| 6.5.3 | Microsoft Office Home & Business 2024 (or most current version) Perpetual License (Mac OS) Perpetual license for Microsoft Office Home & Business 2024 or most current version for macOS; including Microsoft Word, Excel, PowerPoint, Outlook, and OneNote desktop; supported on the most recent macOS version; language support for all EU languages, multiple language interfaces, and proofing tools. |  |  |  |
| 6.5.4 | Acrobat Pro Perpetual License or minimum 12-month license Perpetual license or subscription license with validity not less than 12 months for professional PDF software (e.g., Adobe Acrobat Pro) or equivalent. |  |  |  |
| 6.5.5 | Microsoft Power BI Pro- minimum 12-month license Microsoft Power BI Pro subscription license; validity not less than 12 months. |  |  |  |
| 6.5.6 | Microsoft Power BI Premium Per User- minimum 12-month license Microsoft Power BI Premium Per User subscription license; validity not less than 12 months. |  |  |  |